# Khwopa College of Engineering Libali, Bhaktapur

# Department of Computer Engineering

Computer Network (CT 657)

LAB 5

DHCP

Objective: To configure and verify Dynamic Host Configuration Protocol (DHCP) on a router using Cisco Packet Tracer.

Theory:

DHCP (Dynamic Host Configuration Protocol) is an Application Layer protocol used to automatically assign IP address, subnet mask, default gateway, and DNS server to hosts in a network. DHCP reduces manual configuration errors and simplifies network administration.

DHCP uses UDP ports 67 (server) and 68 (client).

## Procedure(TASK 1)

### Step 1: Create Network Topology

- Open Cisco Packet Tracer

- Place one Router, one Switch, and two PCs

- Connect devices using straight-through cables

Step 2: Configure Router Interface

Router> enable

Router# configure terminal

Router(config)# interface g0/0

Router(config-if)# ip address 192.168.10.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

### Step 3: Configure DHCP on Router

Exclude reserved IP addresses:

Router(config)# ip dhcp excluded-address 192.168.10.1 192.168.10.10

Create DHCP pool:

Router(config)# ip dhcp pool LAN_POOL

Router(dhcp-config)# network 192.168.10.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.10.1

Router(dhcp-config)# dns-server 8.8.8.8

Router(dhcp-config)# exit

### Step 4: Configure PCs as DHCP Clients

- Click PC → Desktop → IP Configuration

- Select DHCP option

- IP address should be assigned automatically

## 5. Verification

On Router:

Router# show ip dhcp binding

Router# show ip dhcp pool

On PC (Command Prompt):

ipconfig

ping 192.168.10.1