# Khwopa College of Engineering Libali, Bhaktapur

# Department of Computer Engineering

Computer Network (CT 657)

LAB 3

Telnet

Objective:

- To study and perform remote login using the Telnet protocol and understand its working in a network environment.

Theory:

Telnet is an application-layer protocol used for remote login over a network. It allows a user to log into another computer on the same network or over the Internet and execute commands remotely.

Telnet operates on TCP port 23 and provides a bidirectional interactive text-based communication facility. Data transmitted using Telnet is unencrypted, making it insecure for sensitive communication.

Because of its security issues, Telnet has largely been replaced by SSH (Secure Shell), but it is still useful for learning networking concepts and testing services.

### Advantages of Telnet

- Simple and easy to use

- Useful for testing network services

- Lightweight protocol

### Disadvantages of Telnet

- No encryption (security risk)

- Vulnerable to eavesdropping

- Not suitable for secure communication

### Applications

- Network device configuration

- Testing TCP services

- Educational and lab environments

### Network Topology

- Client Machine → Telnet Client

- Server Machine → Telnet Server

Both machines must be reachable via IP address.

TASK 1:

### Step 1: Configure PC IP Address

- Click PC → Desktop → IP Configuration

- Assign:

  - IP Address: 192.168.1.2

  - Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.1.1

Step 2:Configure Router Interface

Router> enable

Router# configure terminal

Router(config)# interface g0/0

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Step 3: Configure Telnet on Router

Router(config)# line vty 0 4

Router(config-line)# password cisco

Router(config-line)# login

Router(config-line)# transport input telnet

Router(config-line)# exit

Step4 : for no password set in router (optional)

enable secret class

### Step 4: Access Router Using Telnet

From PC Command Prompt:

telnet 192.168.1.1

TASK 2 : ADD a new PC in a same network and perform telnet