# Khwopa College of Engineering Libali, Bhaktapur

# Department of Computer Engineering

Computer Network (CT 657)

LAB 4

SSH

Objective:To configure and verify Secure Shell (SSH) remote access on a Cisco router using Cisco Packet Tracer.

Theory:

SSH (Secure Shell) is a secure remote login protocol that operates at the Application Layer and uses TCP port 22. Unlike Telnet, SSH encrypts all data including usernames and passwords, making it secure for remote device management.

Precaution :Disable telnet for security.

Task 1 :

### Step 1: Configure IP Address on PC

- Click PC → Desktop → IP Configuration

- Set:

  - IP Address: 192.168.1.2

  - Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.1.1

Step 2: Configure Router Interface

Router> enable

Router# configure terminal

Router(config)# interface fa0/0

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

### Step 3: Configure Router for SSH

#### Set hostname and domain name (mandatory for SSH)

Router(config)# hostname R1

R1(config)# ip domain-name nishan

2. Create local username and password

R1(config)# username admin privilege 15 secret cisco

- Generate RSA keys

R1(config)# crypto key generate rsa

How many bits in the modulus [512]: 1024  (you can choose from 360 to 4096)

- Enable SSH version 2

R1(config)# ip ssh version 2

Step 4: Configure VTY Lines for SSH

R1(config)# line vty 0 4

R1(config-line)# login local

R1(config-line)# transport input ssh

R1(config-line)# exit

## Step:5. Verification

From PC command prompt:

Ping

To access router using SSH:

ssh -l admin 192.168.1.1

R1# sh  ip int br

TASK 2 : Add a new pc in same network and perform SSH .