# Khwopa College of Engineering Libali, Bhaktapur

# Department of Computer Engineering

Computer Network (CT 657)

LAB 2

Basic Routing Configuration

Objective:

- To configure and verify utilizing the CLI to set basic Routing configuration.

- To verify router configurations and network connectivity.

Pre-Requisites:Cisco Packet Tracer

Theory:

Cisco Inter network Operating System (IOS)

  - Cisco Internetwork Operating System is what runs Cisco routers as well as Cisco’s switches.

  - The Cisco Internetwork Operating System (IOS) is the kernel of Cisco routers as well as all current Catalyst switches, hat provides routing, switching, internetworking, and telecommunications features.

  - The first IOS was written by William Yeager in 1986 and enabled networked applications.

  - A Cisco switch is configured using the command-line interface (CLI).

  - You can access the Cisco IOS through the console port of a router or switch, from a modem into the auxiliary (or aux) port on a router, or even through Telnet and Secure Shell (SSH). Access to the IOS command line is called an EXEC session.

  - The console port is usually an RJ-45, 8-pin modular connection located at the back of the device, and there may or may not be a password set on it by default

Bring up a Switch

When you first bring up a Cisco IOS device, it will run a power-on self-test—a POST. Upon passing that, the machine will look for and then load the Cisco IOS from flash memory if an IOS file is present, then expand it into RAM. As you probably know, flash memory is electronically erasable programmable read-only memory—an EEPROM. The next step is for the IOS to locate and load a valid configuration known as the startup-config that will be stored in nonvolatile RAM (NVRAM).

Once the IOS is loaded and up and running, the startup-config will be copied from NVRAM into RAM and from then on referred to as the running-config.

But if a valid startup-config isn’t found in NVRAM, your switch will enter setup mode, giving you a step-by-step dialog to help configure some basic parameters on it.

You can also enter setup mode at any time from the command line by typing the command setup from privileged mode.

Command Line Interface (CLI) Modes of swtich:

  - After the interface status messages appear and you press Enter, the Switch>

prompt will pop up. This is called user exec mode, or user mode for short.

  - Mostly used to view the statistics.

  - It is also a stepping stone to log in to privileged exec mode, called privileged mode for short. Theh Switch# prompt with singal will appear in privileged mode.

  - Privileged mode is used to view and change the switch configuration.

  - The switch are configured into global configuration mode when you need set the router only once and affect the entire router. It makes changes to the running -configuration of a router.

  - By typing config terminal in privileged mode, the switch changes into global configuration mode. The switch prompt appears as Switch(config)#

| Mode | Prompt |
| --- | --- |
| Console Mode | Switch> |
| User exec mode | Switch>enable Switch# |
| Privileged exec mode | Switch#config terminal |

|  | Switch(config)# |
| --- | --- |

You can go back from privileged mode into user mode by using the disable command:

Switch#disable Switch>

You can type logout from either mode to exit the console: Switch>logout

Switch con0 is now available Press RETURN to get started.

Adminstrative Configurations

You can configure the following administrative functions on a router and switch:

  - Hostnames

  - Banners

  - Passwords—enable password ……

Or username …. Password ….

  - Interface Description